Avidyne IFD 4XX / 5XX Wiring and Configuration

Optional diagram to control and display Avidyne com radio on the SkyView screen.

|  | Speed | Data |
| --- | --- | --- |
| In 1 | High | EFIS/Airdata |
| In 2 | High | Off |
| Out 1 | High | GAMA 429 Graphics |
| Out 2 | High | Off |
| SDI | Common |
| VNAV | Enable Labels |

Avidyne MAIN ARINC 429 Settings

Avidyne VOR / LOC / GS ARINC 429 Config

|  | RX | TX |
| --- | --- | --- |
| Speed | High | High |
| Format | Off | VHF429 |
| SDI | Common |
| DME Mode | Directed Freq 1 |

Avidyne Main RS232 Config

|  | Input | Output |  |
| --- | --- | --- | --- |
| CHNL # | VhfCtrl | VhfCtrl | Settings for Avidyne Com radio control |
| CHNL # | Off (may be used for other devices) | Aviation | Setting for SV-ARINC-429 module RS232 input |

Replace # with the port number you wired to the device

SkyView SV-ARINC-429 Settings

| MODULE1 |  |
| --- | --- |
| INPUT SPEED | HIGH |
| INPUT1 | GPS1 |
| NAVIGATION SOURCE DISPL NAME | AVI GPS (name it whatever makes sense to you) |
| INPUT 2 | NAV1 |
| NAVIGATION SOURCE DISP NAME | AVI NAV (name it whatever makes sense to you) |
| OUTPUT SPEED | HIGH |
| ALLOW AUTO SWITCH | YES |

SETUP MENUS > SYSTEM SETUP > ARINC429 SETUP

SkyView RS232 Serial Port Settings for Avidyne Radio Control

| SERIAL IN DEVICE | GARMIN / APOLLO SL-30 |
| --- | --- |

SETUP MENUS > SYSTEM SETUP > SERIAL PORT SETUP > SERIAL PORT #